To acquire videos start all cameras, then run:

chessboard_video

While waving the chessboard around

To calibrate based on videos:

video_path = ‘C:\Users\G71044MH\OneDrive - The University of Manchester\Documents\GitHub\Ethogram\Camera Calibration\calibration01_04_26’;

To align cameras to a specific frame use:

cameraSet  = 1:9;

align_camera_final(video_path, cameraSet);

This saves the camera matrices as Pcal.mat in video_path folder.

To check the 3D works fine can manually label two or more pictures of the lego object.

To do that first label with test_3D_final. For example:

test_3D_final(video_path, 1)

test_3D_final(video_path, 7)

Then test triangulation

transform = triangulate_new_final(video_path, [1 7]);

This also saves transform.mat.

You can then apply transform to the raw 3D data to return them to the standard reference frame coordinates as:

X = transform.b*X*transform.T + transform.c;